Name: Tasneem Mansour Mansour Mansour Farag

ID: 19015538

Professor: Dr. Ahmed Abbas Elserougi

Subject Name: HVDC

Subject Code: EEP 424

Assignment (2) | Bipolar Point-To-Point HVDC-Link | MATLAB Model

Contents

I. System Description:	3

II. Six-Pulse Model:	3

Question (A):	3

Question (B):	5

Model Screenshot:	5

Case (1): 1600MW: Power flow from (A) to (B): Net(A) is rectifier and Net(B) is inverter:	5

Case (2): 1200MW: Power flow from (B) to (A): Net(B) is rectifier (+10%) and Net(A) is inverter (-10%):	8

Case (3): 1600MW: Power flow from (A) to (B): Net(A) is rectifier (2.5% voltage swell) and Net(B) is inverter:	11

III. Twelve-Pulse Model:	15

Question (C):	15

Model Screenshot:	15

Case (1): Power flow from (A) to (B): Net(A) is rectifier and Net(B) is inverter:	15

Case (2): 1200MW: Power flow from (B) to (A): Net(B) is rectifier(+10%) and Net(A) is inverter (-10%):	16

Case (3): 1600MW: Power flow from (A) to (B): Net(A) is rectifier(2.5% voltage swell) and Net(B) is inverter:	17

IV. Comparison:	18

Question (D):	18

V. Attachments:	19

# I. System Description:

- A 1600MW 6-pulse bipolar point-to-point HVDC-link shown below. The link interconnects between two AC networks, (network (A): 22kV/50Hz, network (B):20kV/60Hz), through overhead transmission line with a resistance of 10 Ω. The range of firing angle at the rectifier side is (5deg-25deg), while the inverter is operated under minimum γ of 20deg. 1:10 Y-Y ideal transformers with zero leakage inductances are employed. Proper smoothing reactors (Lsr) are employed as shown.

# II. Six-Pulse Model:

## Question (A):

#### Transferring a power of 1600 MW from side (A) to side (B) during normal operating conditions. Find analytically the suitable firing angle at the rectifier side, and the corresponding dc current.

Vdoi= 32*200πcos20=253.81KV

800=Vdor*Idc

Idc= Vdor-253.8110

∴8000=Vdor2-253.81* Vdor

∴Vdor=282.16KV

Vdor= 32*220πcosα=282.16KV

∴α=18.25°

∴Idc= 282.16-253.8110=2.835KA

#### Transferring a power of 1200 MW from side (B) to side (A) during normal operating conditions. Find analytically the suitable firing angle at the rectifier side, and the corresponding dc current. (Assume -10% tapping at inverter side, and +10% tapping rectifier side).

Vdoi= 32*220*0.9πcos20=251.27KV

600=Vdor*Idc

Idc= Vdor-251.2710

∴6000=Vdor2-251.27* Vdor

∴Vdor=273.23KV

Vdor= 32*200*1.1πcosα=273.23KV

∴α=23.13°

∴Idc= 273.23-251.2710=2.196KA

#### Transferring a power of 1600MW from side (A) to side (B) during 2.5% voltage swell at network (A).

Vdoi= 32*200πcos20=253.81KV

800=Vdor*Idc

Idc= Vdor-253.8110

∴8000=Vdor2-253.81* Vdor

∴Vdor=282.16KV

Vdor= 32*220*1.025πcosα=282.16KV

∴α=22.099°

∴Idc= 282.16-253.8110=2.835KA

## Question (B):

### Model Screenshot:

### Case (1): 1600MW: Power flow from (A) to (B): Net(A) is rectifier and Net(B) is inverter:

#### AC voltage at rectifier side:

#### Vdor: DC voltage at rectifier side:

#### Grid current at rectifier side:

#### AC voltage at inverter side:

#### Vdoi: DC voltage at inverter side:

#### Grid current at inverter side:

#### DC current:

### Case (2): 1200MW: Power flow from (B) to (A): Net(B) is rectifier (+10%) and Net(A) is inverter (-10%):

#### AC voltage at rectifier side:

#### Vdor: DC voltage at rectifier side:

#### Grid current at rectifier side:

#### AC voltage at inverter side:

#### Vdoi: DC voltage at inverter side:

#### Grid current at inverter side:

#### DC current:

### Case (3): 1600MW: Power flow from (A) to (B): Net(A) is rectifier (2.5% voltage swell) and Net(B) is inverter:

#### AC voltage at rectifier side:

#### Vdor: DC voltage at rectifier side:

#### Grid current at rectifier side:

#### AC voltage at inverter side:

#### Vdoi: DC voltage at inverter side:

#### Grid current at inverter side:

#### DC current:

# III. Twelve-Pulse Model:

## Question (C):

### Model Screenshot:

Answer.

### Case (1): Power flow from (A) to (B): Net(A) is rectifier and Net(B) is inverter:

#### AC voltage at rectifier side:

Curve vs time.

#### Vdor: DC voltage at rectifier side:

Curve vs time.

#### Grid current at rectifier side:

Curve vs time.

#### AC voltage at inverter side:

Curve vs time.

#### Vdoi: DC voltage at inverter side:

Curve vs time.

#### Grid current at inverter side:

Curve vs time.

#### DC current:

Curve vs time.

### Case (2): 1200MW: Power flow from (B) to (A): Net(B) is rectifier(+10%) and Net(A) is inverter (-10%):

#### AC voltage at rectifier side:

Curve vs time.

#### Vdor: DC voltage at rectifier side:

Curve vs time.

#### Grid current at rectifier side:

Curve vs time.

#### AC voltage at inverter side:

Curve vs time.

#### Vdoi: DC voltage at inverter side:

Curve vs time.

#### Grid current at inverter side:

Curve vs time.

#### DC current:

Curve vs time.

### Case (3): 1600MW: Power flow from (A) to (B): Net(A) is rectifier(2.5% voltage swell) and Net(B) is inverter:

#### AC voltage at rectifier side:

Curve vs time.

#### Vdor: DC voltage at rectifier side:

Curve vs time.

#### Grid current at rectifier side:

Curve vs time.

#### AC voltage at inverter side:

Curve vs time.

#### Vdoi: DC voltage at inverter side:

Curve vs time.

#### Grid current at inverter side:

Curve vs time.

#### DC current:

Curve vs time.

# IV. Comparison:

### Question (D):

#### Compare the performance of 6-pulse and 12-pulse converters. Comment.

Answer.

# V. Attachments:

- GitHub repo.